CPE301 – SPRING 2020

# Design Assignment 5

The goal of the assignment is to modify the above codes to do the following

- Using the Multi-function shield, using a custom serial transfer protocol display the temperature sensor value (int only) of LM34/35 on to the seven-segment display.

- Using the Multi-function shield, using the SPI protocol display the temperature sensor value (int only) of LM34/35 on to the seven-segment display.

- Using one-wire interface display the temperature sensor for DS18B20 on the UART terminal

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

ATmega328PB Xplained mini		USB cable 		Atmel Studio 7

Data Visualizer 				7 segment display	LM35

Multifunction Shield			DS18B20

TASK 1:							TASK 2:

TASK 3:

Notes

- The lm35 is not working, I talk to Dr. Venki about it, and he told me to just leave a note here.

- For the schematics, I separated the part for the 7 segment because it doesn’t change between task 1 and 2.

- The result of task 1 and 2 are the same: 156 (because lm35 is broken), the demo vids will show that.

- Just to check that my code works, I used an temperature sensor using mux 1000, just to show that code works as it should and its just that the component lm35 is broken.

- There’s no DS18B20 component on fritz so I substitute temp sensor as to represent the part.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Ads

/*

* DA5_T1.c

*

* Created: 4/21/2020 10:09:41 AM

* Author : John Paulo Lumbres

*/

/*LM35 is broken, tried applying direct heat, yet value still doesn't change*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdio.h>

#include <stdlib.h>

#define UBRR_9600 103

#define LATCH_DIO 4 //PD4

#define CLK_DIO 7 //PD7

#define DATA_DIO 0 //PB0

//declaring variable for controlling and storing values

volatile unsigned int adc_val;

//number 0 to 9

const uint8_t segment_map[] ={0xC0, 0xF9, 0xA4, 0xB0, 0x99, 0x92, 0x82, 0xF8, 0X80, 0X90};

//digit select

const uint8_t segment_select[] = {0xF1,0xF2,0xF4,0xF8};

void adc_init(void)

{

//AVcc reference, left adjust, ADC4 = PC4 = LM35,ADC enable

ADMUX =(1<<REFS0)|(1<<ADLAR)|(1<<MUX2);

ADCSRA = (1<<ADEN)|(1<<ADPS2)|(1<<ADPS1);	//prescaler 64

}

void read_adc(void)

{

unsigned char i=4;

adc_val = 0;	//initial value

while(i--){	//get 4 values of ADC

ADCSRA |= (1<<ADSC);

while((ADCSRA & (1<<ADIF))==0);

ADCSRA |= (1<<ADIF);

adc_val+=ADCH;

_delay_ms(50);

}

adc_val = adc_val/4;	//have the average

}

void display(uint8_t sel, uint8_t light){

for(uint8_t i=0; i<8; i++){	//writing data of segment map

if (0 == (light & _BV(7 - i))) {

PORTB &= (0<<DATA_DIO);

} else {

PORTB |= (1<<DATA_DIO);

}

//send pulse to clk dio

PORTD |=(1<<CLK_DIO);

PORTD &=(0<<CLK_DIO);

}

for (uint8_t i=0; i<8; i++){	//writing data of segment select

if (0 == (sel & _BV(7 - i))) {

PORTB &= (0<<DATA_DIO);

} else {

PORTB |= (1<<DATA_DIO);

}

//send pulse to clk dio

PORTD |=(1<<CLK_DIO);

PORTD &=(0<<CLK_DIO);

}

}

int main(void)

{

DDRC &= ~(1<<PC4);	//pc4 as input

PORTC |=(1<<PC4);	//pc4 enabled

DDRB |= (1<<DATA_DIO);	//data_dio is output for portb

DDRD |= (1<<CLK_DIO)|(1<<LATCH_DIO);

//clk_dio and latch_dio are output for portd

adc_init();		//initialize adc

int val1, val2, val3;	//declare int variables

while (1)

{

read_adc();	//read adc val from sensor

//mod adc val to get one's value

val1 = (adc_val)%10;

//divide and mod adc val to get ten's place value

val2 = (adc_val/10)%10;

//divide and mode val to get the hundred's place value

val3 = (adc_val/100)%10;

for(int i=0; i<101;i++){	//repeat 100 times

PORTD &= ~(1<<LATCH_DIO);	//latch to get values

//display one's value on the last 7 segment display

display(segment_select[3],segment_map[val1]);

PORTD |= (1<<LATCH_DIO);	//close latch

_delay_ms(5);

PORTD &= ~(1<<LATCH_DIO);

//display ten's value on the 2nd to the last 7 segment display

display(segment_select[2],segment_map[val2]);

PORTD |= (1<<LATCH_DIO);

_delay_ms(5);

PORTD &= ~(1<<LATCH_DIO);

//display hundred's value on the first 7 segment display

display(segment_select[1],segment_map[val3]);

PORTD |= (1<<LATCH_DIO);

_delay_ms(5);

}

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

As

/*

* DA5_T2.c

*

* Created: 4/21/2020 3:36:49 PM

* Author : John Paulo Lumbres

*/

/*LM35 is broken, tried applying direct heat, yet value still doesn't change*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#define UBRR_9600 103

#define LATCH 2 //PD4

#define CLK 5 //PD7

#define DATA 3 //PB0

#define SHIFT_REGISTER DDRB

#define SHIFT_PORT PORTB

//declaring variable for controlling and storing values

volatile unsigned int adc_val;

//number 0 to 9

const uint8_t segment_map[] ={0xC0, 0xF9, 0xA4, 0xB0, 0x99, 0x92, 0x82, 0xF8, 0X80, 0X90};

//digit select

const uint8_t segment_select[] = {0xF1,0xF2,0xF4,0xF8};

void adc_init(void)

{

//AVcc reference, left adjust, ADC4 = PC4 = LM35,ADC enable

ADMUX =(1<<REFS0)|(1<<ADLAR)|(1<<MUX2);

ADCSRA = (1<<ADEN)|(1<<ADPS2)|(1<<ADPS1);	//prescaler 64

}

void read_adc(void)

{

unsigned char i=4;

adc_val = 0;	//initial value

while(i--){	//get 4 values of ADC

ADCSRA |= (1<<ADSC);

while((ADCSRA & (1<<ADIF))==0);

ADCSRA |= (1<<ADIF);

adc_val+=ADCH;

_delay_ms(50);

}

adc_val = adc_val/4;	//have the average

}

void spi_init(){

//SPI enabled, master mode

SPCR0 |= (1<<SPE)|(1<<MSTR);

//open latch

SHIFT_PORT &= ~(1<<LATCH);

}

void store(){

//opens and close the latch to store

SHIFT_PORT |= (1<<LATCH);

SHIFT_PORT &= ~(1<<LATCH);

}

void display(uint8_t data){

//store data to SPDR0 register

SPDR0 = data;

//send the data to slave

while(!(SPSR0 &(1<<SPIF)));

}

int main(void)

{

DDRC &= ~(1<<PC4);	//PC4 as input

PORTC |=(1<<PC4);	//enable pc4

//have data, latch and clk outputs on portb

SHIFT_REGISTER |= (1<<DATA)|(1<<LATCH)|(1<<CLK);

//initialize portb to 0;

SHIFT_PORT &= ~((1<<DATA)|(1<<LATCH)|(1<<CLK));

adc_init();

spi_init();

int val1, val2, val3;

while (1)

{

read_adc();

//mod adc val to get one's value

val1 = (adc_val)%10;

//divide and mod adc val to get ten's place value

val2 = (adc_val/10)%10;

//divide and mode val to get the hundred's place value

val3 = (adc_val/100)%10;

for (int i=0; i<35000; i++) { //repeat to see display

display(segment_map[val1]);	//send one's value

display(segment_select[3]);	//send which led to turn on

store();

display(segment_map[val2]);	//send ten's value

display(segment_select[2]);	//send which led to turn on

store();

display(segment_map[val3]);	//send hundered's value

display(segment_select[1]);	//send which led to turn on

store();

}

}

}

- DEVELOPED MODIFIED CODE OF TASK 3/A

Main.c

Ds18b20.h

Ds18b20.c

Uart.h

Uart.c

- SCHEMATICS

I’ll be posting the schematic for operating the 7-segments separately, but the connections for sensors and SPI will be done on fritzing.

TASK 1:

TASK 2:

TASK 3:

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Both Task 1 and Task two display value of broken sensor, which is 156. On demo, you will see that both tasks outputs 156. I used internal sensor just to see if it works, and it works, just that the value is in centigrade.

TASK 1 and 2:

using broken sensor			using internal sensor

TASK 3:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

TASK 1:

TASK 2:

Recreated the circuit for a top view look

TASK 3:

Wire is there to connect jumper.

- VIDEO LINKS OF EACH DEMO

Task 1 – https://youtu.be/WoEyfLpDsu4

Task 2 – https://youtu.be/hJMT37sSHkQ

Task 3 – https://youtu.be/5OdyAWBk9A4

- GITHUB LINK OF THIS DA

https://github.com/lumbrj1/submission/tree/master/DesignAssignments

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

John Paulo Lumbres