CPE301 – SPRING 2020

# Midterm 2

Write, simulate, and demonstrate using Atmel Studio 7 a C code for the AVR ATmega328p/pb microcontroller that performs the following functions:

- Use the motor driver, program the ATmega to drive the geared DC motor in CW and CCW direction for a given PWM.

- Using the Potentiometer connected to ADC0, translate the ADC value to PWM value/speed of the motor. Verify the operation.

- Using the CCP capture of PWM1, in mode 1X and 2X determine the speed of the DC Motor for a set ADC pot value/position

- Using CCP capture and interrupt (mode 4x), determine the speed of the DC Motor for a set ADC Pot value/position.

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

ATmega328PB Xplained mini

Data Visualizer

TB6612FNG

USB cable

DC motor

3 AA battery

Atmel Studio 7

BreadBoard

Notes:

- I was able to make encoder B work sometimes, but signal is sometimes unstable.

- I used INT0/INT1 interrupt to read the number of ticks manually

- I picked CCW for direction of motor.

- 1 pushbutton to turn on/off the motor.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* midterm2.c

*

* Created: 4/13/2020 4:06:33 PM

* Author : John Paulo Lumbres

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdio.h>

#include <stdlib.h>

#define MTR_1 5

#define MTR_2 4

#define SW (PINC&(1<<1))

#define UBRR_9600 103

//declaring variable for controlling and storing values

volatile unsigned int power, revCtr0=0, revCtr1=0, escape=0, valcap0=100, valcap1=100;

volatile unsigned int rise0_0, fall0_0, rise0_1, fall0_1;

volatile unsigned int rise1_0, fall1_0, rise1_1, fall1_1;

volatile float t1ovf=0, t2ovf=0, done0=0, done1=0;

volatile float  res1=0, res2=0, res4=0;

volatile float rpm1, rpm2, rpm4;

volatile unsigned int adc_val;

char outs[20];

void adc_init(void)

{

ADMUX =(1<<REFS0)|(1<<ADLAR);	//AVcc reference, left adjust, ADC0 = PC0 = potentiometer,ADC enable

ADCSRA = (1<<ADEN)|(1<<ADPS2)|(1<<ADPS1);	//prescaler 64

}

void read_adc(void)

{

unsigned char i=4;

adc_val = 0;	//initial value

while(i--){	//get 4 values of ADC

ADCSRA |= (1<<ADSC);

while((ADCSRA & (1<<ADIF))==0);

ADCSRA |= (1<<ADIF);

adc_val+=ADCH;

_delay_ms(50);

}

adc_val = adc_val/4;	//have the average

}

void USART_init(unsigned int ubrr) {	//initializes USART

UBRR0H = (unsigned char)(ubrr>>8);	//seting UBBR0

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1<<RXEN0) | (1<<TXEN0);	//enable receiver, transmitter & RX interrupt

UCSR0C = (1<<UCSZ01) | (1<<UCSZ00);	//asynchronous 8 N 1

}

void USART_tx_string(char *data) {	//void to display on terminal

while((*data !='\0')){

while(!(UCSR0A &(1<<UDRE0)));	//while not done reading

UDR0 = *data;

data++;	//keep moving to next bit

}

}

unsigned char uart_receive(void){

while(!(UCSR0A &(1<<7)));	//while rxc0 is not set, keep reading bits

return UDR0;

}

void timer0_init(){

TCCR0A = (1<<COM0A1)|(1<<WGM01)|(1<<WGM00);	//fast PWM

TCCR0B = (1<<CS01);	//prescaler 8

}

void timer1_init(){

TCCR1A = 0;	//normal mode

TCCR1B = (1<<CS11);	//prescaler 8

TIMSK1 = 1;	//overflow interrupt enabled

}

void timer2_init(){

TCCR2A = 0; //normal mode

TCCR2B = (1<<CS21);	//prescaler 8

TIMSK2 = 1;	//overflow interrupt enabled

}

void pc_int(){

PCMSK1 = (1<<PCINT9);	//INT9 (PC1) enabled

PCICR = (1<<PCIE1);	//Pin change interrupt enabled

}

void int_interrupt(){

EIMSK = 0x03;	//INT0 and INT1 external interrupt

EICRA = 0x0f;	//rising edge for both

}

void calculate(){

//factor to multiply ticks by to get rsm

//(1920*8*2)/16000000 = 0.00192

//rpm = 60/rms

_delay_ms(100);

//get average of the 20 values

res1= res1/100;

res2= res2/100;

res4= res4/100;

//using the value and formula calculated above

rpm1 = 60/((float)res1 * 0.00192);

rpm2 = 60/((float)res2 * 0.00192);

rpm4 = 60/((float)res4 * 0.00192);

}

void initial() {

rpm1=rpm2=rpm4=0;	//initialize values

escape=1;

valcap0=20;

valcap1=20;

}

int main(void)

{

int spd_control;	//declare a variable

adc_init();	//initialize adc

USART_init(UBRR_9600);	//initialize usart

timer0_init();	//initialize timers

timer1_init();

timer2_init();

pc_int();	//initialize interrupts

int_interrupt();

DDRC &= ~((1<<PC1)|(1<<PC0));	//PC1 and PC0 are inputs

PORTC = (1<<PC1);	//pull-up enabled

DDRD |= (1<<PD6)|(1<<PD5)|(1<<PD4);	//PD6, PD5, PD4 outputs

DDRD &= ~((1<<PD3)|(1<<PD2));	//PD2, PD3 inputs

PORTD =0;

sei();	//global interrupt enabled

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

D0 is connected to encoder B, D1 is connected to encoder A. As you can see, encoder B only sometimes work which is why there’s inconsistencies on 4x results. The results given above are the few instances where encoder was working for a while.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/ZLLbIZ0vyJM

- GITHUB LINK OF THIS DA

https://github.com/lumbrj1/submission/tree/master/Midterms

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

John Paulo Lumbres